INTERFACE CONTROL DOCUMENT

Static Fire Controller V2  —  Serial & Telnet Telemetry API

| Document Number | ICD-SFC-V2-001 |
| --- | --- |
| Revision | Rev A |
| Date | 22 March 2026 |
| Classification | UNCLASSIFIED / INTERNAL |
| Prepared By | Venkata Sravan G. |
| Team / Division | Avionics – 2026 |
| System | Static Fire Controller V2 (Arduino Mega / Serial-to-WiFi Bridge) |

FOR INTERNAL ROCKETRY TEST USE ONLY

Revision History

| Rev | Date | Author | Description |
| --- | --- | --- | --- |
| A | 22 Mar 2026 | Venkata Sravan G. — Avionics 2026 | Initial release. Covers serial protocol, telemetry CSV, command dictionary, and GUI constraints. |

Table of Contents

| 1 | System Overview | 4 |
| --- | --- | --- |
| 2 | Telemetry Stream (Controller → GUI) | 5 |
| 2.1 | CSV Field Mapping | 5 |
| 2.2 | Thermocouple Fault Parsing (T1–T6) | 6 |
| 2.3 | State Machine Mapping | 7 |
| 3 | Command Dictionary (GUI → Controller) | 8 |
| 3.1 | ARM | 8 |
| 3.2 | DAR | 8 |
| 3.3 | FIR | 9 |
| 3.4 | RST | 9 |
| 3.5 | SLG | 9 |
| 3.6 | CAL 0 / CAL <integer> | 10 |
| 3.7 | TIM <string> | 10 |
| 4 | GUI Development & Logic Constraints | 11 |
| 5 | Acronyms & Definitions | 12 |

1.  System Overview

This Interface Control Document (ICD) defines the complete serial communication protocol for the Static Fire Controller Version 2, used in ground-based rocketry static fire testing. It specifies all telemetry data formats, command structures, acknowledgement sequences, and GUI behavioral constraints required for correct integration.

| Parameter | Value |
| --- | --- |
| Microcontroller | Arduino Mega 2560 (8-bit AVR) |
| Baud Rate | 115,200 bps |
| Data Bits | 8 |
| Parity | None |
| Stop Bits | 1 |
| Line Terminator (TX) | \r\n (CR LF) |
| Line Terminator (RX) | \n (LF) |
| Telemetry Rate | ~10 Hz (approx. every 100 ms) |
| Network Bridge | Serial-to-WiFi / Telnet bridge (TCP socket, same protocol) |

2.  Telemetry Stream  (Controller → GUI)

The controller broadcasts telemetry as a 15-field Comma-Separated Values (CSV) string at approximately 10 Hz. Each frame is terminated with \r\n. The GUI must parse every frame in real-time and must never assume state transitions; it must reflect the State field from the stream.

2.1  CSV Field Mapping

| Index | Field Name | Data Type | Description |
| --- | --- | --- | --- |
| [0] | Timestamp | unsigned long | Microseconds since boot. |
| [1] | Thrust | float | Calibrated load-cell force (engineering units set by CAL). |
| [2] | Pressure | integer | Raw pressure value. Reserved for future use; always 0 in current firmware. |
| [3] | T1 | float / string | Thermocouple 1 — MAX31855. See Section 2.2 for fault string encoding. |
| [4] | T2 | float / string | Thermocouple 2 — MAX31855. See Section 2.2. |
| [5] | T3 | float / string | Thermocouple 3 — MAX31855. See Section 2.2. |
| [6] | T4 | float / string | Thermocouple 4 — MAX31855. See Section 2.2. |
| [7] | T5 | float / string | Thermocouple 5 — MAX31855. See Section 2.2. |
| [8] | T6 | float / string | Thermocouple 6 — MAX31855. See Section 2.2. |
| [9] | T7 | float | Analog Temperature Sensor 1 (degrees C). |
| [10] | T8 | float | Analog Temperature Sensor 2 (degrees C). |
| [11] | T9 | float | Analog Temperature Sensor 3 (degrees C). |
| [12] | T10 | float | Analog Temperature Sensor 4 (degrees C). |
| [13] | T11 | float | Board Internal Temperature (degrees C). |
| [14] | State | integer | Current System State Enum (0–4). See Section 2.3. |

2.2  Thermocouple Fault Encoding  (Fields [3]–[8])

Fields T1 through T6 use MAX31855 hardware sensors with built-in open/short-circuit fault detection. The GUI parser MUST handle dynamic types for these six fields. The parser shall treat any non-numeric token as a fault code and display it accordingly.

| Fault String Literal | Fault Meaning |
| --- | --- |
| OPEN | Thermocouple wire is disconnected or not present. |
| Short GND | Thermocouple lead is shorted to ground potential. |
| Short VCC | Thermocouple lead is shorted to the supply voltage rail. |

NOTE:  The exact string literals above (including capitalisation and space) must be matched verbatim. Any deviation indicates a firmware change; update the GUI parser accordingly.

2.3  State Machine Mapping  (Field [14])

| Value | Enum Name | Description |
| --- | --- | --- |
| 0 | SYSTEM_SAFE | Safe / de-energised standby state. Continuity loss forces this state instantly. |
| 1 | SYSTEM_IDLE | System idle and awaiting the ARM command. |
| 2 | SYSTEM_ARMED | Armed; SD card logging active. Awaiting FIR command. |
| 3 | SYSTEM_IGNITION | Pyrotechnic outputs active. Automatically transitions to state 4 after exactly 5 seconds. |
| 4 | SYSTEM_POST_IGNITION | Post-fire; logging active. Awaiting RST command to return to SYSTEM_SAFE. |

NOTE:  If continuity hardware detects a drop below the hardcoded threshold, the controller will force State = 0 (SYSTEM_SAFE) immediately, regardless of the current state.

3.  Command Dictionary  (GUI → Controller)

All commands sent from the GUI to the controller must be terminated with a single newline character (\n). Upon receiving a recognised command the controller replies with the ACK responses listed below, always terminated with \r\n. Commands sent in invalid states are silently ignored.

3.1  ARM

| Command Token | ARM |
| --- | --- |
| Action | Transitions the system state from SYSTEM_IDLE (1) to SYSTEM_ARMED (2) and initiates SD card data logging. |
| Precondition | State must equal 1 (SYSTEM_IDLE). Command is rejected in all other states. |
| Response | ACK Transition successful; logging has started. |
| Response | FAULT: SD OPEN FAILED SD card could not be opened; state remains IDLE. |

3.2  DAR

| Command Token | DAR |
| --- | --- |
| Action | Disarms the system. Transitions state to SYSTEM_IDLE (1) and deactivates pyrotechnic safety circuitry. |
| Precondition | Accepted in any state. |
| Response | ACK Disarm successful. |

3.3  FIR

| Command Token | FIR |
| --- | --- |
| Action | Fires the pyrotechnic initiators. Transitions state from SYSTEM_ARMED (2) to SYSTEM_IGNITION (3). After exactly 5 seconds the firmware automatically transitions to SYSTEM_POST_IGNITION (4). No GUI stop command is required. |
| Precondition | State must equal 2 (SYSTEM_ARMED). Command is rejected in all other states. |
| Response | ACK Fire sequence initiated. |

3.4  RST

| Command Token | RST |
| --- | --- |
| Action | Resets the system. Transitions state from SYSTEM_POST_IGNITION (4) to SYSTEM_SAFE (0). |
| Precondition | State must equal 4 (SYSTEM_POST_IGNITION). |
| Response | ACK System reset to SYSTEM_SAFE. |

3.5  SLG

| Command Token | SLG |
| --- | --- |
| Action | Stops SD card logging manually and flushes / closes the log file. |
| Precondition | Anytime when SD logging is active. |
| Response | ACK Logging stopped and file closed. |

3.6  CAL 0  /  CAL <integer>

| Command Token | CAL 0 / CAL <integer> |
| --- | --- |
| Action | Two-step calibration procedure. (1) CAL 0 — tares the load cell by recording the current raw ADC reading as the zero offset. (2) CAL <N> — sets the scale factor so the current thrust reading equals the integer N. Must be performed after CAL 0 with a known reference mass applied. |
| Precondition | Accepted in any state. |
| Response | Offset set to: <value>\r\nACK Response to CAL 0. |
| Response | Scale set to: <value>\r\nACK Response to CAL <integer> (whole integers only). |

3.7  TIM <string>

| Command Token | TIM <string> |
| --- | --- |
| Action | Injects a named timestamp marker into the SD card data log. The string may include underscores (e.g., TIM Test_Ignition_Start). Spaces in the string are not guaranteed to be preserved by the firmware. |
| Precondition | Anytime. |
| Response | Time Stamp:\r\n<string>\r\nACK Marker successfully written to log. |

4.  GUI Development & Logic Constraints

The following constraints are mandatory for any GUI implementation that interfaces with the Static Fire Controller V2. Violation of these constraints may result in unsafe operational behaviour.

| ID | Title | Requirement |
| --- | --- | --- |
| C-01 | State Mirroring | The GUI shall derive its displayed state exclusively from the State field (index [14]) in the received telemetry stream. The GUI must not assume a command was successful based solely on receiving an ACK. State transitions may be overridden by hardware events at any time. |
| C-02 | Continuity Override | If the controller detects that pyro continuity has dropped below the hardcoded threshold, it will immediately and autonomously force State = 0 (SYSTEM_SAFE) without any GUI interaction. The GUI must reflect this transition within one telemetry cycle. |
| C-03 | Igniter Timer — Hardware Controlled | The 5-second ignition duration is enforced entirely in firmware after the FIR command is accepted. The GUI must not send any 'stop fire' command. The GUI need only monitor the telemetry stream until State transitions from 3 to 4. |
| C-04 | Dynamic Type Parser Required | The GUI CSV parser must handle dynamic types for fields [3]–[8] (T1–T6). A robust parser shall first attempt float conversion; if the conversion fails, it shall treat the value as a fault code string and display it with appropriate visual indication. |
| C-05 | Command Acknowledgement Timeout | The GUI should implement a reasonable ACK timeout (recommended: 500 ms). If no ACK is received within the timeout, the GUI should display a communication warning and not assume the command was executed. |
| C-06 | SD Card Fault Handling | If the ARM command returns 'FAULT: SD OPEN FAILED', the system remains in SYSTEM_IDLE. The GUI must alert the operator and prevent progression toward FIR until a successful ARM acknowledgement is received. |

5.  Acronyms & Definitions

| Acronym / Term | Definition |
| --- | --- |
| ACK | Acknowledgement |
| AVR | Alf and Vegard's RISC processor (microcontroller architecture) |
| bps | Bits per second |
| CAL | Calibration command |
| CSV | Comma-Separated Values |
| DAR | Disarm command |
| DXA | Document Extended Attribute (unit: 1/20 of a typographic point = 1/1440 inch) |
| FIR | Fire command |
| GUI | Graphical User Interface |
| ICD | Interface Control Document |
| MAX31855 | Thermocouple-to-digital converter IC with SPI interface (Maxim Integrated) |
| RST | Reset command |
| SD | Secure Digital (memory card) |
| SFC | Static Fire Controller |
| SLG | Stop Logging command |
| TCP | Transmission Control Protocol |
| TIM | Timestamp marker command |
| VCC | Voltage at the Common Collector (supply voltage rail) |
| WiFi | Wireless Fidelity (IEEE 802.11) |